Log of errata in IEC 61131-3 (2002) (informal)

| Date | Document |
| --- | --- |
| 2003-05-07 |  |

| Source | Clause/ Subclause | Paragraph/ Figure/ Table | Type of comment (General/ Technical/Editorial) | COMMENTS | Proposed change | OBSERVATIONS OF THE SECRETARIAT on each comment submitted |
| --- | --- | --- | --- | --- | --- | --- |
| TC65/WG6 | 1.3 |  | Technical | Missing definition: variable. | ADD the definition of variable from IEC 61499-1. | Received 2003-01-20. |
| TC65/WG6 | 2.3.3.2 | Table 12 | Editorial | Table 12 is referenced in 2.3.3.1 but appears in 2.3.3.2. | MOVE Table 12 to subclause 2.3.3.1. | Received 2003-01-20. |
| TC65/WG6 | 2.3.3.3 | Various | Technical | Rules for assignment of values of aggregates and "anonymous" enumerated types are not clear. | Include information similar to subclause 3.1.2, 3.1.7 and 3.1.9 of IEC 61131-8, 2nd Ed. | Received 2003-01-21. |
| TC65/WG6 | 2.4.1.2 | Various | Technical | Rules for assignment of values of aggregates are not clear. | Include information similar to subclause 3.1.7 and 3.1.9 of IEC 61131-8, 2nd Ed. | Received 2003-01-21. |
| TC65/WG6 | 2.6.5 | Fig. 17a | Technical | "x = 0 or 1" in the caption may be misinterpreted as "x = (0 OR 1)", i.e. "x = 1" | CHANGE to "x = (don't care), i.e. x{0,1}" | Received 2003-01-20. |
| TC65/WG6 | 2.6.4.5 | NOTE 5 | Technical | This NOTE may be misinterpreted as allowing both options (with/without "final scan") to be implemented simultaneously. | CHANGE"which" to "which one" | Received 2003-01-20. |
| TC65/WG6 | 2.6.4.5 | Table 45a | Technical | This table may be misinterpreted as allowing both options (with/without "final scan") to be implemented simultaneously. | ADD "NOTE - These two features are mutually exclusive, i.e., only one of the two shall be supported in a given SFC implementation." | Received 2003-01-20. |
| TC65/WG6 | 2.5.2.2 | Table 33 | Technical | "Description" is inconsistent with "Example" in items 2c, 2d. | CHANGE RETAIN to NON_RETAIN in "Description" column | Received 2003-01-20. |
| TC65/WG6 | 2.5.2.3.3 | Table 36 | Editorial | CTUD_UDINT is missing from Features 3a-3d. | ADD CTUD_UDINT as Feature 3d, renumber CTUD_ULINT as Feature 3e | Received 2003-01-20. |
| TC65/WG6 | 2.4.3.2 | Table 18, Feature 7 | Editorial | Array notation is not correctly represented. | CHANGE ARRAY(1..2,1..3) to ARRAY[1..2,1..3] | Received 2003-01-20. |
| TC3 of PLCopen | 2.5.1.2 | Table 20 | Technical | Since EN/ENO has to be supported in LD to allow e.g. arithmetic functions, the support of LD automatically forces a vendor to support EN/ENO in all languages (according to Note a in Table 20). | Is this behaviour intended? TC3 of PLCopen believes that this is an error in the Standard. | Received 2003-05-07. |